Jun, 2019

Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, based on MasterFormat 2016 and The Project Resource Manual—CSI Manual of Practice. The Manufacturer is responsible for technical accuracy.

The section must be carefully reviewed and edited by the Architect or Engineer to meet the requirements of the project and local building code.  Words and sentences within brackets [ ] are choices to include or exclude a particular item or statement.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” after editing this section.

Video Surveillance

IP Cameras

NETWORK CAMERA

- – GENERAL

  - SUMMARY

    - Section Includes

      - Video Surveillance – Surveillance Cameras – Camera Housings

      - Video Surveillance – Surveillance Cameras – Camera Mounts

      - Video Surveillance – Video Surveillance Sensors

    - Related Sections

      - [Security Detection, Alarm and Monitoring – Security Monitoring and Control – Security Monitoring and Control Software].

**********Specifier’s note: Include those standards referenced elsewhere in this SECTION.

  - REFERENCES

| STANDARD | SUPPORT OR NOT |
| --- | --- |
| CE-LVD: EN62368-1 | √ |
| CE-EMC: Electromagnetic Compatibility Directive 2014/30/EU | √ |
| CE-RED: Radio Equipment Directive 2014/53/EU |  |
| FCC: 47 CFR FCC Part 15, Subpart B | √ |
| FCC ID: FCC PART 15C |  |
| UL/CUL: UL60950-1 CAN/CSA C22.2 No.60950-1-07 |  |
| EN50155 |  |
| E-MARK: ECE Regulation 10.05 |  |

  - SYSTEM DESCRIPTION

    - Section Includes

      - Video Surveillance – Surveillance Cameras – IP Cameras

  - SUBMITTALS

    - Product Data:

      - Manufacturer’s data, user and installation manuals for all equipment and software programs including computer equipment and other equipment required for complete video management system.

    - Dimensional Drawings; include

      - Overall device dimensions.

      - Dimensions specific for installation.

    - Closeout Submittals

      - User manual.

      - Parts list.

      - Maintenance requirements.

  - QUALITY ASSURANCE

    - Manufacturer:

      - Minimum of [10] years of experience in manufacture and design Video Surveillance Devices.

    - Video Surveillance System:

      - List certifying bodies (UL, etc.)

      - Provide evidence of compliance upon request.

    - Installer:

      - Minimum of [5] years of experience installing Video Surveillance System.

  - DELIVERY, STORAGE AND HANDLING

    - Comply with requirements of Section 01 60 00.

    - Deliver materials in manufacture’s original, unopened, undamaged containers; and unharmed original identification labels.

    - Protect store materials from environmental and temperature conditions following manufacturer’s instructions.

    - Handle and operate products and systems according to manufacturer’s instructions.

  - WARRANTY

    - Provide manufacturer’s warranty covering [3] years for replacement and repair of defective equipment. Warranty varies country to country.

  - MAINTENANCE

    - Make ordering of new equipment for expansions, replacements, and spare parts available to dealers and end users.

    - Provide factory direct technical support via phone and e-mail.

- – PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer:

Zhejiang Dahua Vision Technology Co., Ltd.

Address: No.1199 Bin’an Road, Binjiang District, Hangzhou, China

Tel: +86-571-87688883

Fax: +86-571-87688815

Email:overseas@dahuasecurity.com

    - Substitutions:

      - [All proposed substitutions must be approved by the Architect or Engineer professional.]

      - [Proposed substitutions must provide a line-by-line compliance documentation.]

  - 2MP Lite Full-colour Fixed-focal Bullet Network Camera – [DH-IPC-HFW2230M-AS-LED-B] [DH-IPC-HFW2230M-AS-LED-B]

| Image Sensor | 1/2.8”2Megapixel progressive CMOS |
| --- | --- |
| Pixel | 2MP |
| Effective Pixels | 1920 (H) ×1080 (V) |
| Min. Illumination | 0.01 Lux@F1.6 |
| Focal Length | 3.6 mm; 6 mm |
| Max. Aperture | 3.6 mm: F1.6 6 mm: F1.6 |
| Field of View | 3.6 mm: Pan 87° × Tilt 46° × Diagonal 104° 6 mm: Pan 53° × Tilt 30° × Diagonal 60° |
| Video Compression | H.265; H.264; H.264B; MJPEG |
| Video Frame Rate | Main stream: 1920 × 1080 (1 fps–25/30 fps) Sub stream: 704 × 576 (1 fps–25 fps) 704 × 480 (1 fps–30 fps) |
| Resolution | 1080p (1920 × 1080); 1.3M (1280 × 960); 720p (1280 × 720); D1 (704 × 576/704 × 480); VGA (640 × 480); CIF (352 × 288/352 × 240) |
| Bit Rate Control | CBR/VBR |
| Day/Night | Color/B/W |
| BLC | Yes |
| HLC | Yes |
| WDR | DWDR |
| White Balance | Auto/natural/street lamp/outdoor/manual/regional custom |
| Noise Reduction | 3D DNR |
| Region of Interest (RoI) | Yes (4 areas) |
| Image Rotation | 0°/90°/180°/270°(Supports 90°/270° with 1080p resolution and lower.) |
| Privacy Masking | 4 areas |
| Audio Interface | Yes |
| Built-in MIC | NA |
| Audio Compression | G.711a; G.711Mu; G.726; AAC |
| Alarm | Yes |
| Network | RJ-45 (10/100 Base-T) |
| Protocol | IPv4; IPv6; HTTP; HTTPS; TCP; UDP; ARP; RTP; RTSP; RTCP; RTMP; SMTP; FTP; SFTP; DHCP; DNS; DDNS; QoS; UPnP; NTP; Multicast; ICMP; IGMP; NFS; Bonjour |
| Interoperability | ONVIF (Profile S/Profile G); CGI; P2P; Genetec |
| Edge Storage | FTP; SFTP; Micro SD card (support max. 128 G); NAS |
| Management Software | Smart PSS; DSS; DMSS |
| Mobile Phone | IOS; Android |
| Micro SD Card (Max.) | 128 G |
| Audio Input | 1 channel (RAC port) |
| Audio Output | 1 channel (RAC port) |
| Alarm Input | 2 channels in: 5mA 3V–5V DC |
| Alarm Output | 2 channel Out: 300mA 12V DC |
| Power Supply | 12V DC/PoE (802.3af) |
| Operating Temperature | –40°C to +60°C (–40°F to +140°F） |
| Storage Temperature | –40°C to +60°C (–40°F to +140°F） |
| Ingress Protection | IP67 |
| Casing | Metal + plastic |
| Installation | Wall mount; ceiling mount |

- – EXECUTION

  - EXAMINATION

    - Examine areas to receive devices and notify adverse conditions affecting installation or subsequent operation.

    - Do not begin installation until unacceptable conditions are corrected.

  - PREPARATION

    - Protect devices from damage during construction.

    - Install devices in accordance with manufacturer’s instruction at locations indicated on the floor drawings plans.

    - Perform installation with qualified service personnel.

    - Install devices in accordance with the National Electrical Code or applicable local codes.

    - Ensure selected location is secure and offers protection from accidental damage.

    - Location must provide reasonable temperature and humidity conditions, free from sources of electrical and electromagnetic interference.

  - FIELD QUALITY CONTROL

    - Test snugness of mounting screws of all installed equipment.

    - Test proper operation of all video system devices.

    - Determine and report all problems to the manufacturer’s customer service department.

  - ADJUSTING

    - Make proper adjustment to video system devices for correct operation in accordance with manufacturer’s instructions.

    - Make any adjustment of camera settings to comply with specific customer’s need.

  - DEMOSTRATION

    - Demonstrate at final inspection that video management system and devices functions properly.

END OF SECTION